16-bit vs 8-bit: Ensure you save as 16-bit TIFF if you plan to do quantitative analysis; 8-bit TIFFs lose data for intensity measurements.

Z-Stacks: If your .lif is a 3D Z-stack, saving it as a TIFF will usually create a multi-page TIFF (one stack in one file). Some software prefers you to export it as an "Image Sequence" (one TIFF file per slice

Method 1 (Manual) :Using Fiji / ImageJ (Best for Researchers)

Fiji is the gold standard for this because it handles the complex metadata correctly.

- Drag and drop your .lif file into the Fiji status bar.

- The Bio-Formats Import Options window will appear.

- Select the series (images) you want to open and click OK.

- Once the image is open, go to File > Save As > Tiff...

Method 2 (automatic): Batch Converting (Best for many files)

2.1 Fiji Macro: You can use a built-in or custom macro (like Lif2Tiff) to automate the process. These macros can automatically split channels and save them as individual TIFFs.

2.2 Command Line: For power users, the "bfconvert" tool from the Bio-Formats command line interface can convert large datasets without opening the Fiji GUI.

2. Fiji macro code (javascript):

Copy the below code into a new script window in Fiji (Plugins > New > Macro):

// This macro batch converts .lif files in a folder to .tif

// It preserves spatial scaling and metadata automatically.

run("Bio-Formats Macro Extensions");

setBatchMode(true); // Runs in the background to save time

input = getDirectory("folder location containing .lif files");

output = getDirectory("destination folder to store .tiff");

list = getFileList(input);

for (i = 0; i < list.length; i++) {

if (endsWith(list[i], ".lif")) {

processFile(input, output, list[i]);

}

}

function processFile(input, output, file) {

path = input + file;

Ext.setId(path);

Ext.getSeriesCount(seriesCount); // .lif files often contain many image series

for (s = 0; s < seriesCount; s++) {

// Open each series individually

run("Bio-Formats Importer", "open=[" + path + "] color_mode=Default view=[Standard ImageJ] stack_order=XYZCT series_" + (s+1));

title = getTitle();

// CLEAN THE FILENAME ---

title = getTitle();

// Remove spaces and slashes which cause IOErrors

cleanTitle = replace(title, " ", "_");

cleanTitle = replace(cleanTitle, "/", "-");

saveAs("Tiff", output + title + ".tif");

close();

}

}

setBatchMode(false);

print("I have completed converting all .lif files to .tiff");

extra:

// Optional: Uncomment the next line if you want to burn a scale bar into the image pixels

// run("Scale Bar...", "width=10 height=4 font=12 color=White background=None location=[Lower Right] bold overlay");

Note: consider using Lif2Tiff, a specialized ImageJ macro that includes a user interface for choosing whether to split channels or maintain stacks.

2.1 Modify it to perform Maximum Intensity Projections on Z-stacks automatically.

(TBC)

2.2 Help you split channels into separate folders.

(TBC)